Kitchen Cabinets Design Specification

December 18, 2019

Sink Wall

Cabinets

- Three-drawer cabinet – 32.5” W x 34.5” H x 24” D

- Pull-out garbage cabinet - 21” W x 34.5” H x 24” D

- Sink cabinet (centered under windows) – 36” W x 34.5” H x 24” D

- Three-drawer cabinet – 29.5” W x 34.5” H x 24” D

Electrical Needs

- Dishwasher, garbage disposal outlet & switch

- Outlets above countertops

- Lights centered above each window, and switch

Stove Wall

Cabinets

- Two upper cabinets with two fixed shelves and two adjustable shelves each –

42” W x 64” H x 12” D

- One upper cabinet above range hood – 36” W x 22” H x 12” D

- Range hood – 36” W x 20.25” D

- Two lower cabinets with three drawers each – 45” W x 34.5” H x 24” D

Electrical Needs

- Vent hood outlet – 88” up from floor, 63” from wall on left

- Outlets above countertops

- Under cabinet lights & switch

Refrigerator Wall                                                 Pantry Shelves

Refrigerator opening requirement:

36” W x 71 ½” H

Cabinets

- Cabinet above refrigerator with pull-out shelf – 36” W x 24.5” H x 24” D

- Cabinet above refrigerator pull-out – 36” W x 22” H x 24” D

- Three-drawer cabinet – 33.5” W x 34.5” H x 24” D

- Three open shelves – 33.5” W x 3” H x 12” D

Electrical Needs

- Refrigerator outlet & coffee pot outlet

- Three or four outlets in pantry (at “above countertop” level)

- Under cabinet light & switch in coffee bar area

- Light & switch in pantry

Island

Kitchen Side

Seating Side

Cabinets

- One cabinet with one drawer and cookie sheet storage – 30” W x 34.5” H x 24” D

- Two cabinets with one drawer and two pull-out shelves – 30” W x 34.5” H x 24” D

- Three cabinets with three pull-out shelves – 30” W x 34.5” W x 24” D

15” overhang of countertop on seating side

Electrical Needs

- Outlets on both sides of island

- Two pendant lights centered above island, and switch

Kitchen Layout – Electrical Needs

Kitchen electrical needs summary

- Garbage disposal outlet & switch

- Dishwasher

- Vent hood outlet – 88” up from floor, 63” from wall on left

- Island – outlets on both sides

- Refrigerator outlet

- Outlets above all countertops

- Three or four outlets in walk-in pantry at countertop level

- Lights & switches in ceiling, pantry, above island, and over each sink window

- Under cabinet lights on both sides of stove and coffee bar area with switches

Kitchen Layout – Cabinet Needs

Kitchen cabinet summary

- Nineteen (19) cabinets

- Twenty-nine (29) doors

- Twenty-one (21) drawer fronts

- Nineteen (19) drawer boxes

- Thirteen (13) pull-out shelves (lower cabinets)

- Two (2) ¾” side panels for refrigerator cabinets that go to floor

- Two (2) decorative end panels for island

- One (1) 36” shaker vent hood cover

- One (1) pull-out garbage next to sink

- One (1) pull-out over refrigerator

- One (1) cookie sheet divider section in island

- One (1) spice drawer insert for drawer on stove wall (optional)

- One (1) 24” deep counter for microwave in pantry

- One (1) 24” deep pantry shelf below counter

- Five (5) 12” deep pantry shelves

- Crown molding at 10’ around upper cabinets

- Trim piece at 8’ level around upper cabinets to hide cabinet seams

- Soft-close door hinges

- Soft-close drawer slides, 100 lb. capacity per pair

Laundry Room

Laundry Room - Cabinets

Cabinets

- Two cabinets over washer and dryer – 30” W x 42” H x 12” D each

- Two drawers in desk area – 20.5” W x 4.5” H x 22” D each

Supports for butcher block countertop, with opening for desk area and laundry sorters

Laundry Room – Layout & Countertop

Washer

Dryer